Comprehensive Operations Analysis (COA)

Technical Memorandum #6/7 – Short / Long Range Service Plan Recommendations

October 10, 2014

In Association With:

Submitted by:

Submittee by:

# Table of Contents

1.0	Introduction	1

2.0	Identification of Inter-Related Service Improvement Packages	2

2.1	Inter-Related Service Improvement Packages	2

2.2	Service Package Isolation	2

2.3	Service Improvement Prioritization	3

3.0	Short-Range Service Plan	4

3.1	Short-Range 1-5 Year Service Plan	4

3.2	Operating Requirements	41

4.0	Long Range Service Plan	48

4.1	Long Range 6-10 Year Service Plan	48

4.2	Operating Requirements	68

5.0	COA Vehicle and Facility Needs	73

5.1	COA Vehicle Needs	73

5.2	Transit Passenger Facility Needs	73

5.3	New Maintenance Facility	75

5.4	Scheduling Software / CAD/AVL Needs	75

# List of Tables

Table 3.1: COA Short Range Service Plan by Implementation Year	4

Table 3.1: COA Short Range Service Plan by Implementation Year (continued)	5

Table 3.2: Short Range Plan Proposed Service Hours and Frequencies	24

Table 3.2: Short Range Plan Proposed Service Hours and Frequencies (continued)	25

Table 3.3: COA Short Range Service Plan Daily Revenue Hours	41

Table 3.4: COA Short Range Service Plan Daily Revenue Miles	41

Table 3.5: COA Short Range Service Plan Annual Revenue Hours	41

Table 3.6: COA Short Range Service Plan Annual Revenue Miles	41

Table 3.7: COA Short Range Service Frequency and Operating Requirements by day of Week (continued)	45

Table 4.1: COA Long Range Service Plan by Implementation Year	48

Table 4.2: Long Range Plan Proposed Service Hours and Frequencies	57

Table 4.2: Long Range Plan Proposed Service Hours and Frequencies (continued)	58

Table 4.3: COA Long Range Service Plan Daily Revenue Hours	68

Table 4.4: COA Long Range Service Plan Daily Revenue Miles	68

Table 4.5: COA Long Range Service Plan Annual Revenue Hours	68

Table 4.6: COA Long Range Service Plan Annual Revenue Miles	68

Table 4.7: COA Long Range Service Frequency and Operating Requirements by day of Week	69

Table 4.7: COA Long Range Service Frequency and Operating Requirements by day of Week (continued)	70

Table 5.1: COA Peak Vehicle Requirements	73

# List of Figures

Figure 3.1: Short Range Weekday Proposed RTS Transit Network	26

Figure 3.2: Short Range Weekday Proposed RTS Transit Network – New Segments Added	27

Figure 3.3: Short Range Weekday Proposed RTS Transit Network – Existing Segments Eliminated	28

Figure 3.4: Short Range Weekday Proposed RTS University of Florida Campus Transit Network	29

Figure 3.5: Short Range Weekday Proposed RTS University of Florida Campus Transit Network – New Segments Added	29

Figure 3.6: Short Range Weekday Proposed RTS University of Florida Campus Transit Network – Existing Segments Eliminated	31

Figure 3.7: Short Range Saturday Proposed RTS Transit Network	32

Figure 3.8: Short Range Saturday Proposed RTS Transit Network – New Segments Added	33

Figure 3.9: Short Range Saturday Proposed RTS Transit Network – Existing Segments Eliminated	34

Figure 3.10: Short Range Weekend Proposed RTS University of Florida Campus Transit Network	35

Figure 3.11: Short Range Weekend Proposed RTS University of Florida Campus Transit Network – New Segments Added	36

Figure 3.12: Short Range Weekend Proposed RTS University of Florida Campus Transit Network – Existing Segments Eliminated	37

Figure 3.13: Short Range Sunday Proposed RTS Transit Network	38

Figure 3.14: Short Range Sunday Proposed RTS Transit Network – New Segments Added	39

Figure 3.15: Short Range Sunday Proposed RTS Transit Network – Existing Segments Eliminated	40

Table 3.7: COA Short Range Service Frequency and Operating Requirements by day of Week	42

Figure 4.1: Long Range Weekday Proposed RTS Transit Network	59

Figure 4.2: Long Range Weekday Proposed RTS Transit Network – New Segments Added	60

Figure 4.3: Long Range Weekday Proposed RTS Transit Network – Existing Segments Eliminated	61

Figure 4.4: Long Range Saturday Proposed RTS Transit Network	62

Figure 4.5: Long Range Saturday Proposed RTS Transit Network – New Segments Added	63

Figure 4.6: Long Range Saturday Proposed RTS Transit Network – Existing Segments Eliminated	64

Figure 4.7: Long Range Sunday Proposed RTS Transit Network	65

Figure 4.8: Long Range Sunday Proposed RTS Transit Network – New Segments Added	66

Figure 4.9: Long Range Sunday Proposed RTS Transit Network – Existing Segments Eliminated	67

# Appendicies

Appendix A: Inter-related Service Improvements

Appendix B: Service Improvement Packages

Introduction

This Technical Memorandum presents specific Short Range and Long Range service recommendations for the Gainesville Regional Transit System (RTS) Comprehensive Operations Analysis (COA). These recommendations have been prepared after completion of the following project tasks:

- Extensive fieldwork and service operations;

- Completion of a 50% on-board survey of weekday bus service;

- A market and latent demand analysis;

- Extensive public outreach efforts including interviews with various stakeholder groups, RTS staff interviews and driver sessions, public meetings and rider input obtained at the Rosa Park downtown transit center, the Oaks Mall and Butler Plaza; and

- An extensive evaluation of system-level and route-level performance, including the completion of individual route profiles.

Existing route performance characteristics, route strengths and weaknesses were discussed with RTS staff.   Service recommendations are based on those discussions.

The Short-Range and Long Range plans are presented in this Tech Memo reflecting service time periods of 1 to 5 years and 6 to 10 years, respectively.  The Short-Range plan recommendations are designed to optimize the existing service structure with minimal service expansion, realizing an approximate 6.7% increase in overall annual service-hours over Spring 2014 service levels.  The Long-Range Plan reflects a more significant increase in service-hours associated with route structure changes and improved service frequency and span of service hours, resulting in a 12.8% increase in overall annual service-hours over Spring 2014 service levels.  Recommendations in the Short Range Plan exceed RTS current financial resources, but are included in the COA for identifying specific service proposals, should additional funding resources become available.

The following section in this Technical Memorandum (Section 2.0) presents details on the inter-related service changes and will assist in the identification of implementation packages of improvements which are required to be implemented together.  Section 3.0 and 4.0 present anticipated service requirements for COA recommendations, detailed by Short-Range (1-5 years) and Long-Range (6-10 years) planning horizons. Section 5.0 presents an assessment of vehicle and facility needs.

Identification of Inter-Related Service Improvement Packages

The RTS COA Study service plan recommendations start with the overall vision for service improvements (Long-Range Plan). This vision was then prioritized for implementation based on community desires, affordability, and adaptability into the existing RTS system, how the improvements are interrelated and require joint implementation, as well as a proper sequencing or phasing of improvements. The following sub-sections describe the process for which interrelated service improvements were packaged together for joint implementation as well as the prioritization of those improvements over the ensuing ten year timeframe.

  - Inter-Related Service Improvement Packages

Overall transit system performance is not driven by individual route performance, however it is impacted by the overall network of transit services, how they interact with each other, and the efficiency for which users of the service can traverse the transit system network for their mobility needs. Therefore, the focus of the COA service recommendations were on developing a transit system composed of effective and efficient transit routes. Proposed COA Study service improvements were designed to work together to create a long term system that improves overall Gainesville mobility.

Service improvements identified by the COA Study include changes related to:

- Route alignments

- Service frequency

- Span of service hours (e.g., later evening service)

- Days of service (e.g., added weekend service)

- Future transit facility locations as well as existing facility capacity

- Elimination of route segments or entire routes (partially restored by other routes)

- Addition of new route segment service on existing routes

- Addition of new routes

Many of the changes identified above require joint implementation. Factors which influence the packaging of service changes include: route segment elimination and/or replacement by different routes, interlining or linking together of routes to optimize use of resources (vehicles), and the ability to maintain service frequency and span of service hours along specific route segments transferred between routes. Appendix A contains a list of the first cut at inter-related service improvements. This list is advanced further in the following section.

  - Service Package Isolation

From the list of inter-related service improvements, the study team developed an initial process to evaluate these improvements. In addition to grouping the inter-related service improvements, the process examined these improvements across four factors to assist the team in prioritizing the improvements. These four factors include:

- Does the improvement involve additional cost? yes or no

- Does the improvement result in a net cost savings? yes or no

- Was the improvement in the initial list of improvements identified for a future Sur-Tax? yes or no

- Was the improvement identified as a need by survey respondents in surveys conducted with RTS transit users (conducted at Oaks Mall, Butler Plaza and Rosa Parks Terminal on May 1, 2014) and public meeting attendees (also held on May 1, 2014)? yes or no

The inter-related improvements were then packaged together by service day (i.e., weekday, Saturday or Sunday) identifying each inter-related service improvement item number.  The list of Service Improvement Packages, with response to prioritization factors and item numbers assigned are contained in Appendix B.

  - Service Improvement Prioritization

From the Service Improvement Packages list, the study team utilized the four factors for prioritization (listed above) plus input from RTS staff regarding projected average annual hour allocations by year (supported by projected Sur-Tax revenues) to prioritize the list of service improvement packages across a ten year period.   The prioritized list of improvements, along with daily and annual hour estimates, is contained in Appendix C. This list was further refined in the following two sections which detail the improvements proposed for the Short-Range Service Plan (Year 1-5) and Long-Range Service Plan (Year 6-10).

Short-Range Service Plan

  - Short-Range 1-5 Year Service Plan

The short range portion of the COA service plan (years 1-5) focuses on improving mobility for customers currently served by RTS fixed-route service.  Emphasis is placed on improving connectivity at neighborhood transit centers and connection points (e.g. Waldo Road Walmart, Oaks Mall, Northside Walmart, Butler Plaza, etc.).  This is done through a combination of alignment changes as well as frequency adjustments that employ the use of clock-face headways.  This allows routes to connect more regularly throughout the day.  Service on the UF campus is also modified to provide additional circulation within the campus core.  New service areas are also explored in north Gainesville in addition to a new express route from the apartment communities along Southwest 27th Avenue to the university.  Rounding out the short range plans are improvements to service frequencies on some of RTS’ more popular routes plus new weekend and evening services.

Table 3.1: COA Short Range Service Plan by Implementation Year

Table 3.1: COA Short Range Service Plan by Implementation Year (continued)

- Route Level Service Plan Recommendations

The following is a brief description of COA proposed service changes by route. Each description identifies proposed recommendations for changes to route alignment, service frequency, and span of service hours for weekday, Saturday and Sunday. Route descriptions are categorized as City Routes, UF Campus Routes and Later Gator Routes. Short-Range Service Plan (Year 1-5) changes are not recommended on the following routes: 1, 17, 20, 23, 39, 43, 62, 76 and 77. Table 3.2 identifies proposed Short-Range Plan service hours and frequencies by day of week. Figures 3.1 through 3.15 depict the proposed Short-Range Service Plan Network, new segments added and existing segments eliminated by day of week for system wide and UF Campus services.

City Routes

Route 2 – Downtown Station to NE Walmart Supercenter via Lincoln Estates

Route Alignment Recommendations:

Route 2 alignment modified in the Lincoln Estates area, travels north on SE 15th Street, east on SE 12th Avenue, north on SE 19th Terrace, east on SE 8th Avenue, continuing north on SE 24th Street to University Avenue along existing route to the Waldo Road Walmart Supercenter (southbound routing reversed)

Service Frequency Recommendations:

Weekday: no change

Saturday: no change

Sunday: new service 60 minute frequencies

Span of Service Recommendations:

Weekday: no change

Saturday: no change

Sunday: new service

Route 5 – Downtown Station to Oaks Mall via University Avenue

Route Alignment Recommendations:

No alignment changes are recommended.

Service Frequency Recommendations:

Weekday: improve afternoon peak frequency from 24 to 20 minutes

Saturday: no change

Sunday: improve service from 60 to 30 minute frequencies all day

Span of Service Recommendations:

Weekday: no change

Saturday: no change

Sunday: no change

Route 6 – Downtown Station to North Walmart Supercenter via 6th Street

Route Alignment Recommendations:

The COA route alignment recommendation is to streamline this route to operate along NW 6th Street and extend it to the North Walmart Supercenter. The route 6 is proposed to exit the Rosa Park RTS Downtown Station, travel west on SW Depot Avenue, north on NW 6th Street, west on NW 45th Avenue, north on NW 13th Street, south on NW 22nd Street to the North Walmart Supercenter. (southbound operates along reverse alignment)

Service Frequency Recommendations:

Weekday: no change

Saturday: improve service frequencies all day from 120 to 60 minutes

Sunday: new service at 60 minutes

Span of Service Recommendations:

Weekday: no change

Saturday: no change

Sunday: new service

Route 7 – Downtown Station to Eastwood Meadows

Route 7 is recommended to be eliminated and replaced with new branch on the existing Route 11 (refer to Route 11 description below).

Route 8 – Shands to North Walmart Supercenter via NW 13th Street

Route Alignment Recommendations:

The COA route alignment recommendation is to interline the Route 8 and 13 together through the UF Campus on Saturdays and Sundays during the Short-Range Time Period.

Service Frequency Recommendations:

Weekday: no change

Saturday: improve all day service from 90 to 30 minutes

Sunday: Improve all day service from 90 to 45 minutes

Span of Service Recommendations:

Weekday: no change

Saturday: begin 25 minutes later in morning and extend 2 hours later in the evening hours

Sunday: begin 2 hours earlier in the morning and extend 45 minutes later in the evening hours

Route 9 – Reitz Union to Hunters Run

Route Alignment Recommendations:

Route 9 alignment is recommended to operate along Archer Road versus Mowry Road between SW 23rd Drive and Center Drive. Mowry Road will continue to be served by existing Routes 36 and 117 as well as modified Route 121(see below).

Service Frequency Recommendations:

Weekday: no change

Saturday: service is recommended to be deleted and replaced with modified Route 35

Sunday: service is recommended to be deleted and replaced with modified Route 35

Span of Service Recommendations:

Weekday: service is recommended to operate 1 hour earlier in the late evening hours.

Saturday: service is recommended to be deleted and replaced with modified Route 35

Sunday: service is recommended to be deleted and replaced with modified Route 35

Route 10 – Downtown Station to Santa Fe via 16th/23rd Avenue

Route Alignment Recommendations:

No alignment changes are recommended.

Service Frequency Recommendations:

Weekday: no change

Saturday: improve service frequencies all day from 120 to 60 minutes

Sunday: no service

Span of Service Recommendations:

Weekday: no change

Saturday: no change

Sunday: no service

Route 11 – Downtown Station to Eastwood Meadows via University/SE Hawthorne

Route Alignment Recommendations:

The existing Route 11 would continue to Eastwood Meadows as route pattern 11A at its current 60 minute frequencies. This route would be enhanced with a second route pattern, Route 11B, which would operate along the same alignment as Route pattern 11A between the Rosa Parks RTS Downtown Station and the intersection of NE 25th Street and University Avenue.  At this location the Route 11B pattern would travel west on University Avenue, south on SE 24th Street following the existing Route 7 alignment to Eastwood Meadows.  Each Pattern would operate 60 minute frequency resulting in a combined service frequency of 30 minutes along common route segments. Pattern 11B would be interlined with the Route 15 to ensure sufficient schedule recovery time between the two routes.

Service Frequency Recommendations:

Weekday: operate 30 minutes on combined pattern 11A and 11B segments, 60 minutes on individual pattern segments

Saturday: new 60 minute service on pattern 11B (former Route 7 alignment), maintain 60 minutes on 11A pattern

Sunday: new 60 minute service on pattern 11B (former Route 7 alignment), maintain 60 minutes on 11A pattern

Span of Service Recommendations:

Weekday: extend pattern 11A one hour later in evening, extend pattern 11B (former Route 7) two hours later in the evening

Saturday: operate new service on pattern 11B (no service currently operates on former Route 7) for 13 hours

Sunday: operate new service on pattern 11B (no service currently operates on former Route 7) for 10.5 hours

Route 12 – Reitz Union to Butler Plaza

Route Alignment Recommendations:

No alignment changes are recommended.

Service Frequency Recommendations:

Weekday: improve late evening service from 45 minutes to 23 minutes

Saturday: improve afternoon and evening service from 45 minutes to 23 minutes

Sunday: no change

Span of Service Recommendations:

Weekday: no change

Saturday: extend 2 hours later in the evening

Sunday: no change

Route 13 – NW 23rd Street to FloridaWorks via UF / 13th Street

Route Alignment Recommendations:

Route 13 alignment is recommended to be extended to serve a greater portion of the 13th Street corridor.  Currently this route serves SW 13th Street from the UF Campus south to FloridaWorks. The proposed alignment would extend this route north on NW 13th Street to NE 23rd Street.  This route would continue to serve the UF Campus and the Shands Hospital Complex. The COA route alignment recommendation is to interline the Route 8 and 13 together through the UF Campus on Saturdays and Sundays during the Short-Range Time Period.

Service Frequency Recommendations:

Weekday: reduce peak and midday frequency to 15 minutes from 10 minutes (existing frequency not warranted), reduce evening frequency to 35 minutes from 30 minutes (resulting from cycle time expansion)

Saturday: improve frequencies from 60 to 30 minutes all day

Sunday: improve frequencies from 60 to 45 minutes all day

Span of Service Recommendations:

Weekday: no change

Saturday: extend 2 hours later into the evening

Sunday: operate 1 hour earlier in the morning and extend 1 hour later in the evening

Route 15 – Downtown Station to NW 13th Street

Route Alignment Recommendations:

Route 15 currently operates a large one-way loop at it northern end of the route alignment. The COA Study recommends to reduce the loop operations and operate two-way service along NW 13th Street north of 23rd Avenue. A small loop would continue to be operated as a turnaround at the end of this route, operating south on NW 13th Street, west on NW 16th Avenue, north on NW 16th Terrace, east on NW 23rd Avenue, and north on NW 13th Street, continuing the reverse routing.

Service Frequency Recommendations:

Weekday: improve evening service from 60 minutes to 45 minute frequencies

Saturday: improve service from 60 to 30 minute frequency all day

Sunday: improve service from 60 to 30 minute frequency all day

Span of Service Recommendations:

Weekday: start 30 minutes later in the morning and extend service 2 hours later in the evening

Saturday: extend 2 hours later in the evening

Sunday: operate 2 hour earlier in the morning and extend service 1 hour later in the evening

Route 16 – UF / Shands to Sugar Hill

Route Alignment Recommendations:

No changes are proposed to the existing Route 16 alignment during the Short-Range Plan time period.

Service Frequency Recommendations:

Weekday: no change

Saturday: improve frequency from 60 to 30 minutes

Sunday: improve frequency from 60 to 30 minutes

Span of Service Recommendations:

Weekday: no change

Saturday: no change

Sunday: no change

Route 21 – Reitz Union to Cabana Beach via 20th Avenue

Route Alignment Recommendations:

No alignment changes are recommended.

Service Frequency Recommendations:

Weekday: improve peak frequency from 25 to 10 minutes

Saturday: no service

Sunday: no service

Span of Service Recommendations:

Weekday: reduce morning service by 1.5 hours, start at 8 a.m.

Saturday: no service

Sunday: no service

Route 24 – Downtown Station to Lamplighter / Job Corps

Route Alignment Recommendations:

The COA recommends route alignment changes to the Route 24 to streamline its operations in a more north-south orientation along 15th Street. The proposed routing would operate from the Rosa Parks RTS Downtown Station, travel east on SE 7th Avenue, north on SE 15th Street, east on NE 8th Avenue, north on NE 19th Terrace, through the Waldo Walmart Supercenter and NE Waldo Road, west on NE 16th Avenue, north on NE 15th Street to NE 39th Avenue.  At this location the route would operate two route patterns.  Route 24A (Pattern 1) would operate east and southeast along NE 39th Avenue to the FDOT Operations Center (same as existing Route 24A). Route 24B (Pattern 2) would operate north on NE 15th Street, east on NE 53rd Avenue, south on NE Waldo Road, east on NE 49th Avenue, north on NE 40th Terrace, west on NE 54th Avenue, south on NE Waldo Road, returning west on NE 53rd Avenue in the reverse inbound route alignment.

Service Frequency Recommendations:

Weekday: no change

Saturday: no service

Sunday: no service

Span of Service Recommendations:

Weekday: no change

Saturday: no service

Sunday: no service

Route 25 – UF Commuter Lot to Airport

Route Alignment Recommendations:

This route will continue to provide service between the Gainesville Airport and the UF Commuter on weekdays and the UF Cultural Plaza on Saturday and Sunday, however the route alignment between these locations is proposed to change. Route 25 will depart the Airport, travel west on NE 39th Avenue, south on NE Waldo Road, south on SE 11th Street, west on SE 4th Avenue, south on SE 3rd Street, serve the Rosa Parks RTS Downtown Station, travel west on SW Depot Avenue, north on SW 11th Street, west on SW 9th Road and SW Archer Road.  On Weekdays, service would enter the UF Campus via north on Center Drive, travel west on Mowry Road and north on Gale Lemerand Drive to the UF Commuter Lot Stop. On Weekends, service would enter the UF Campus via Center Drive, travel west on Mowry Road and Hull Road to the UF Cultural Plaza.

Service Frequency Recommendations:

Weekday: improve service from 65 to 60 minute frequency

Saturday: no change

Sunday: no change

Span of Service Recommendations:

Weekday: no change

Saturday: no change

Sunday: no change

Route 27 – Downtown Station to 16th Avenue / Health Department

Route Alignment Recommendations:

Route 27 currently operates very minimal service levels on weekdays only connecting the Rosa Parks RTS Downtown Station to the Downtown Santa Fe College Campus, Greyhound Station and the Waldo Road Walmart. The Route 27 has been expanded to assume a larger role in providing improved east–west connectivity in the area north and northeast of the downtown area. The COA recommends the following route alignment. From the Rosa Parks RTS Downtown Station, this route would travel west on SW Depot Avenue, north on Main Street, west on University Avenue (northbound direction), north on NW 1st Street, west on NW 1st Avenue (note: this routes operates NW 1st Avenue to Main Street south in the southbound direction), north on NW 5th Street through the Santa Fe College Downtown Campus, west on NW 5th Avenue, north on NW 6th Street, east on NW/NE 8th Avenue, north on NE 9th Street, east on NE 16th Avenue, south on NE Waldo Road, through the Waldo Road Walmart Supercenter, south on NE 19th Terrace, east on NE 8th Avenue, south on NE 25th Street, west on University Avenue, and south on SE 24th Street, ending at the Health Department.

Service Frequency Recommendations:

Weekday: no change, continue to operate 60 minutes on extended service hours

Saturday: no service

Sunday: no service

Span of Service Recommendations:

Weekday: extend afternoon service 3 hours later until 6:00 p.m.

Saturday: no service

Sunday: no service

Route 28 – The Hub to Butler Plaza via 20th Avenue

Route Alignment Recommendations:

The COA Study recommends the following route alignment change to the Route 28. This route would continue to operate as it currently does between The Hub on UF Campus and the intersection of SW 20th Avenue and SW 38th Terrace. At this point the route would continue south on SW 38th Terrace and its future extension to SW 42nd Street south to the new Transit Center, once completed.

Service Frequency Recommendations:

Weekday: extend frequency from 16 to 20 minutes

Saturday: no service

Sunday: no service

Span of Service Recommendations:

Weekday: no change

Saturday: no service

Sunday: no service

Route 34 – The Hub to 34th Street Post Office / Lexington Crossing

Route Alignment Recommendations:

The COA Study recommends the following route alignment change to the Route 34. This route would continue to serve from The Hub on the UF campus to SW 34th Street and Archer Road.  From this intersection the Route 34 is proposed to operate south on SW 34th Street, deviate to serve the US Post Office, continue south, then northeast on SW Williston Road, north on SW 25th Terrace, west on SW 40th Place, north on SW 27th Street, East on SW 38th Place and south on 26th Drive to SW 40th Place. Extension of this route beyond the Post Office would be ied to anticipated development along the south side of Williston Road.

Service Frequency Recommendations:

Weekday: increase frequency from 20 to 25 minutes all day, exception evening increase from 25 to 38 minutes

Saturday: no service

Sunday: no service

Span of Service Recommendations:

Weekday: reduce evening service by 1 hour

Saturday: no service

Sunday: no service

Route 35 – Reitz Union to 35th Place

Route Alignment Recommendations:

The COA Study recommends the following route alignment change to the Route 35. This route would continue to serve from Reitz Union on the UF campus to SW 23rd Street and SW 35th Place.  From this intersection the Route 35 is proposed to operate west on SW 35th Place, south on SW 27th Street, east on SW 40th Place, south on SW 25th Terrace and southwest on SW Williston Road, continuing along the existing route alignment.

Service Frequency Recommendations:

Weekday: no change

Saturday: improve service frequencies from 45 to 23 minute frequency all day

Sunday: improve service frequencies from 45 to 23 minute frequency all day

Span of Service Recommendations:

Weekday: no change

Saturday: extend evening hours by 3 hours

Sunday: operate service 1 hour earlier in the morning, extend evening by 1 hour

Route 36 – Reitz Union to Butler Plaza via SW 35th Place

Route Alignment Recommendations:

The COA Study recommends the following route alignment change to the Route 36. This route would start on the UF Campus at Reitz Union, travel south on Center Drive, west on Mowry Road, south on SW 23rd Drive, southwest on Archer Road, south on SW 23rd Terrace / SW 23rd Street, west on SW 35th Place, south on SW 34th Street, west and north on SW 39th Boulevard / SW 37th Boulevard, northeast on Windmeadows Boulevard, southeast on SW 35th Boulevard to the Butler Plaza Transit Stop. Reverse routing departs Butler Plaza south east on SW 35th Boulevard, southwest on Archer Road and south / east on SW 37th Boulevard following the reverse of the outbound route alignment.

Service Frequency Recommendations:

Weekday: no change

Saturday: no service

Sunday: no service

Span of Service Recommendations:

Weekday: no change

Saturday: no service

Sunday: no service

Route 38 – Reitz Union to Gainesville Place

Route Alignment Recommendations:

No alignment changes are recommended.

Service Frequency Recommendations:

Weekday: improve morning peak frequency from 26 to 13 minutes

Saturday: no service

Sunday: no service

Span of Service Recommendations:

Weekday: no change

Saturday: no service

Sunday: no service

Route 41 – Downtown Station to North Walmart Supercenter

Route Alignment Recommendations:

The COA Study recommends the following route alignment change to the Route 41. This route would start from the Walmart Supercenter located on NW 34th Street near US 441, travel south on NW 34th Street, east on NW 53rd Avenue, south on NW 13th Street, east on NE 23rd Avenue, south on Main Street, east on SW Depot Avenue to the Rosa Parks RTS Downtown Station.

Service Frequency Recommendations:

Weekday: operate 60 minute frequency all day

Saturday: no service

Sunday: no service

Span of Service Recommendations:

Weekday: no change

Saturday: no service

Sunday: no service

Route 46 – Reitz Union to Downtown Station

Route Alignment Recommendations:

The COA Study recommends the existing Route 46 route alignment to be changed to reflect bidirectional service along the proposed streetcar alignment (Streetcar Study). The proposed Route 46 alignment would begin at the Rosa Parks RTS Downtown Station, travel north on SE 3rd Street, west on SE/SW 2nd Avenue, enter the UF Campus along Union Road, south on Newell Drive to the Reitz Union transit stop.

Service Frequency Recommendations:

Weekday: improve morning peak hours from 30 to 15 minute frequency, the rest of the day remains the same as existing

Saturday: no service

Sunday: no service

Span of Service Recommendations:

Weekday: no change

Saturday: no service

Sunday: no service

Route 53 – NW 43rd Street to North Walmart Supercenter / North US Hwy 441

Route Alignment Recommendations:

The COA Study recommends the new Route 53 to operate between Thomebrook Village (near the intersection of NW 43rd Street and NW 16th Boulevard) to north US 441 and NW 43rd Street. From the intersection of NW 25th Avenue and NW 43rd Street, this route would travel east and south on NW 25th Avenue and NW 41st Street, northwest on NW 16th Boulevard, north on NW 43rd Street, east on NW 53rd Avenue, north on NW 34th Street serve the Walmart Supercenter, continue northwest on US 441 to NW 43rd Street.

Service Frequency Recommendations:

Weekday: new service at 60 minutes all day

Saturday: no service

Sunday: no service

Span of Service Recommendations:

Weekday: new service between 8:00 a.m. and 6:00 p.m.

Saturday: no service

Sunday: no service

Route 75 – Oaks Mall to Butler Plaza via 75th Street

Route Alignment Recommendations:

The COA Study recommends the following route alignment change to the Route 75.  Route 75 would continue to provide service between the Oaks Mall and Butler Plaza. The proposed route alignment change requires the completion of the roadway work designed to connect SW 8th Avenue with NW 61st Street. Once this connection is complete, the Route 75 would operate south on SW 75th Street from West University Avenue, travel east on SW 8th Avenue / NW 61st Street, west on SW 20th / 24th Avenue, and proceed south on SW 75th Street along it existing alignment to Butler Plaza.   The COA Study also recommends b-directional service along the Tower Circle deviation from SW 75th Street along NW 4th Boulevard and West University Avenue. Bus service to the segment between NW 4th Boulevard and West University Avenue would be eliminated and served by bus stops located at each of these signalized intersections.

Service Frequency Recommendations:

Weekday:  improve peak period frequency from 35 to 30 minutes, improve midday frequency from 53 to 30 minutes, extend evening frequencies from 35 to 45 minutes

Saturday: improve service frequency from 105 to 45 minutes all day

Sunday: new service at 45 minute frequency

Span of Service Recommendations:

Weekday: extend evening service 3.5 hours later

Saturday: begin morning service 20 minutes later (7:00 a.m.), extend evening service 1.5 hours later

Sunday: new service between the hours of 8:00 a.m. and 6:30 p.m.

Route 227 – Campus Lodge to Reitz Union

Route Alignment Recommendations:

The COA Study recommends the new Route 227 to operate between SW 27th Street and Reitz Union on UF Campus. RTS experiences significant passenger boardings along SW 27th Street and SW 35th Place resulting in overloads to the existing Routes 9, 35 and 36 in this area. To relieve these over loads, the Route 227 is designed to operate during peak student loading times in the morning hours, focused on arrives to class start times at UF.  The Route 227 would operate local stop pick-up along SW 27th Street and SW 35th Place and operate closed door non-stop to Reitz Union.

Service Frequency Recommendations:

Weekday: approximately 33 minute frequency designed to meet UF class start times (including some walk time on campus)

Saturday: no service

Sunday: no service

Span of Service Recommendations:

Weekday: new route service between 8:10 a.m. and 10:18 a.m. only

Saturday: no service

Sunday: no service

UF Campus Routes

The COA Study recommends a change in the manner in which UF Campus routes serve the central core of the campus. A common route pattern is recommended, serving The Hub and Rawlings Hall. UF circulator routes serving various parts of the campus would converge and traverse the core loop alignment in a clockwise manner.  The loop begins at The Hub, travels north on Buchman Drive, east on Union Road, south on Newell Drive, west on Museum Road, north on Gale Lemerand Drive, and east on Stadium Road returning to The Hub.

Route 116 – University Avenue / 2nd Avenue

Route Alignment Recommendations:

The COA Study recommends the new Route 116, providing circulator service between The Hub and University Avenue and SW 2nd Avenue. The Route 116 would depart The Hub, travel north on Buckman Drive, west on University Avenue, south on SW 34th Street, east on SW 2nd Avenue, south on Village Drive, east on West Fraternity Row / Fraternity Row / Stadium Road returning to The Hub.

Service Frequency Recommendations:

Weekday: new service at 25 minutes all day

Saturday: no service

Sunday: no service

Span of Service Recommendations:

Weekday: new route service between 7:00 a.m. and 6:00 p.m.

Saturday: no service

Sunday: no service

Route 117 – Park & Ride 2 (34th Street)

Route Alignment Recommendations:

The Route 117 is proposed to operate along its existing alignment from Park-N-Ride Lot 2, east on Hull Road / Mowry Road, north on Center Drive, west on Museum Road, north on Gale Lemerand Drive, and east on Stadium Road, serve the Hub, north on Buchman Drive, east on Union Road, south on Newell Drive, west on Museum Road, south on Center Drive returning reverse route to Park-N-Ride Lot 2.

Service Frequency Recommendations:

Weekday: improve service frequency from 16 to 15 minutes all day

Saturday: no service

Sunday: no service

Span of Service Recommendations:

Weekday: no change

Saturday: no service

Sunday: no service

Route 118 – Park & Ride #1 (Cultural Center)

Route Alignment Recommendations:

The COA Study recommends a change in the manner in which UF Campus routes serve the central core of the campus. A common route pattern is recommended, serving The Hub and Rawlings Hall. UF circulator routes serving various parts of the campus would converge and traverse the core loop alignment in a clockwise manner.  The loop begins at The Hub, travels north on Buchman Drive, east on Union Road, south on Newell Drive, west on Museum Road, north on Gale Lemerand Drive, and east on Stadium Road returning to The Hub. The Route 118 is proposed to operate along its existing alignment from Park-N-Ride Lot 1 to the intersection of Museum Drive and Gale Lemerand Drive.  At this location the Route 118 would travel north on Gale Lemerand Drive, and east on Stadium Road, serve the Hub, north on Buchman Drive, east on Union Road, south on Newell Drive, west on Museum Road, returning to Park-N-Ride #1 via the existing routing.

Service Frequency Recommendations:

Weekday: no change

Saturday: no service

Sunday: no service

Span of Service Recommendations:

Weekday: no change

Saturday: no service

Sunday: no service

Route 119 – Family Housing

Route Alignment Recommendations:

The existing Route 119 connects Family Housing with The Hub.  The COA Study proposes the elimination of this route and its replacement with a modified Route 125 (see description below).

Service Frequency Recommendations:

Weekday: route eliminated and replaced by modified Route 125

Saturday: no service

Sunday: no service

Span of Service Recommendations:

Weekday: route eliminated and replaced by modified Route 125

Saturday: no service

Sunday: no service

Route 120 – West Circulator / Fraternity Row

Route Alignment Recommendations:

No alignment changes are recommended.

Service Frequency Recommendations:

Weekday: extend frequencies from 9 to 10 minutes all day

Saturday: no service

Sunday: no service

Span of Service Recommendations:

Weekday: no change

Saturday: no service

Sunday: no service

Route 121 – North-South Campus

Route Alignment Recommendations:

The COA Study recommends a restructuring of the Route 121 and 122, resulting in parts of the existing Route 122 to be served by the new Route 121 alignment.  The new Route 121 service begins at The Hub, travels north on Buchman Drive / NW 17th Street, east on NW 5th Avenue, north on NW 12th Street, west on NW 7th Avenue, south on NW 17th Street / Buchman Drive, west on Stadium Drive, south on Gale Lemerand Drive, east on Mowry Road, south on Center Drive, southwest on SW Archer Road, serve the Shands Parking garage #3 and the Shands Cair Heliport, exit Sheely Drive westbound on SW 16th Avenue, merge into SW Archer Road, north on SW 23rd Drive, east on Mowry Drive, north on Gale Lemerand Drive, east on Stadium Road returning to The Hub.

Service Frequency Recommendations:

Weekday: extend service frequency from 15 to 23 minutes during the day and 30 to 45 in the evening.

Saturday: no service

Sunday: no service

Span of Service Recommendations:

Weekday: no change

Saturday: no service

Sunday: no service

Route 122 – Veterinary School / Shands

Route Alignment Recommendations:

As noted above under Route 121, portions of the existing Route 122 are replaced with a restructured Route 121.  The new Route 122 is designed to connect the UF core campus to the Veterinary School.  The new Route 122 service begins at The Hub, travels north on Buchman Drive, east on Union Drive, south on Newell Drive, southwest on SW Archer Road, south on SW 16th Street, west on SW 16th Avenue, south on Shealy Drive to the Veterinary School, returning north on Shealy Drive, northeast on SW Archer Road, north on Center Drive (to closer serve Shands Hospital), west on Mowry Road, north on Gale Lemerand Drive, east on Stadium Road to The Hub.

Service Frequency Recommendations:

Weekday: improve service frequency from 45 to 30 all day.

Saturday: no service

Sunday: no service

Span of Service Recommendations:

Weekday: no change

Saturday: no service

Sunday: no service

Route 125 – Lakeside / Fraternity Row

Route Alignment Recommendations:

The COA Study proposes a new route alignment for the Route 125. The new alignment would provide connections between The Hub, Fraternity Row and Family Housing.  From The Hub, the route would travel north on Buchman Drive, west on University Avenue, south on Gale Lemerand Drive, west on Stadium Road, west and south on Fraternity Row, west on Museum Road, west on Radio Road, south on Bledsoe Drive, south through Maguire Village and University Village, east on Hull Road,  returning north on Bledsoe Drive, east on Radio Road, east on Museum Road, north and west on Fraternity Row and east on Stadium Drive returning to The Hub.

Service Frequency Recommendations:

Weekday: no change

Saturday: new service at 25 minutes all day

Sunday: new service at 25 minutes all day

Span of Service Recommendations:

Weekday: extend service until 1:00 a.m. (replacing route 126)

Saturday: operate interlined with Route 127 between 10:00 a.m. and 9:00 p.m.

Sunday: operate interlined with Route 127 between 10:00 a.m. and 10:00 p.m.

Route 126 – UF East/West Circulator

Route Alignment Recommendations:

The COA Study recommends this route to be eliminated and replaced with modified Routes 125 and 127.

Service Frequency Recommendations:

Weekday: eliminate route

Saturday: eliminate route

Sunday: eliminate route

Span of Service Recommendations:

Weekday: eliminate route

Saturday: eliminate route

Sunday: eliminate route

Route 127 – Sorority Row / Beaty Towers

Route Alignment Recommendations:

The COA Study recommends a change to Route 127 alignment on UF Campus. The proposed route alignment would travel west on Museum Road, north on Gale Lemerand Drive, east on Stadium Road to The Hub (continue as Route 125 during times when these two routes are interlined), travel north on Buchman Drive, and east on Union Drive following the existing Route 127 alignment.

Service Frequency Recommendations:

Weekday: improve service frequencies from 12 to 10 minutes during the day, 24 to 10 minutes in the evening and extend service in the late evenings at 25 minute frequency

Saturday: new service at 25 minutes all day

Sunday: new service at 25 minutes all day

Span of Service Recommendations:

Weekday: extend service until 1:00 a.m. (replacing route 126)

Saturday: operate interlined with Route 125 between 10:00 a.m. and 9:00 p.m.

Sunday: operate interlined with Route 125 between 10:00 a.m. and 10:00 p.m.

UF Later Gator

Route 300 – Later Gator A (Frat Row)

Route Alignment Recommendations:

No alignment changes are recommended.

Service Frequency Recommendations:

Weekday: no change

Saturday: no change

Sunday: no service

Span of Service Recommendations:

Weekday: begin service 30 minutes later in the evening (9:00 p.m. versus 8:30 p.m.)

Saturday: begin service 30 minutes later in the evening (9:00 p.m. versus 8:30 p.m.)

Sunday: no service

Route 301 – Later Gator B (Southwest Gainesville)

Route Alignment Recommendations:

No alignment changes are recommended.

Service Frequency Recommendations:

Weekday: no change

Saturday: no change

Sunday: no service

Span of Service Recommendations:

Weekday: begin service 30 minutes later in the evening (9:00 p.m. versus 8:30 p.m.)

Saturday: begin service 30 minutes later in the evening (9:00 p.m. versus 8:30 p.m.)

Sunday: no service

Route 302 – Later Gator C (Oaks Mall)

Route Alignment Recommendations:

No alignment changes are recommended.

Service Frequency Recommendations:

Weekday: no change

Saturday: no change

Sunday: no service

Span of Service Recommendations:

Weekday: begin service 30 minutes later in the evening (9:00 p.m. versus 8:30 p.m.)

Saturday: begin service 30 minutes later in the evening (9:00 p.m. versus 8:30 p.m.)

Sunday: no service

Route 303 – Later Gator D (NW 13th Avenue)

Route Alignment Recommendations:

No alignment changes are recommended.

Service Frequency Recommendations:

Weekday: no change

Saturday: no change

Sunday: no service

Span of Service Recommendations:

Weekday: begin service 30 minutes later in the evening (9:00 p.m. versus 8:30 p.m.)

Saturday: begin service 30 minutes later in the evening (9:00 p.m. versus 8:30 p.m.)

Sunday: no service

Route 305 – Later Gator F (Butler Plaza)

Route Alignment Recommendations:

No alignment changes are recommended.

Service Frequency Recommendations:

Weekday: no change

Saturday: no change

Sunday: no service

Span of Service Recommendations:

Weekday: begin service 30 minutes later in the evening (9:00 p.m. versus 8:30 p.m.)

Saturday: begin service 30 minutes later in the evening (9:00 p.m. versus 8:30 p.m.)

Sunday: no service

Table 3.2: Short Range Plan Proposed Service Hours and Frequencies

Table 3.2: Short Range Plan Proposed Service Hours and Frequencies (continued)

Figure 3.1: Short Range Weekday Proposed RTS Transit Network

Figure 3.2: Short Range Weekday Proposed RTS Transit Network – New Segments Added

Figure 3.3: Short Range Weekday Proposed RTS Transit Network – Existing Segments Eliminated

Figure 3.4: Short Range Weekday Proposed RTS University of Florida Campus Transit Network

Figure 3.5: Short Range Weekday Proposed RTS University of Florida Campus Transit Network – New Segments Added

Figure 3.6: Short Range Weekday Proposed RTS University of Florida Campus Transit Network – Existing Segments Eliminated

Figure 3.7: Short Range Saturday Proposed RTS Transit Network

Figure 3.8: Short Range Saturday Proposed RTS Transit Network – New Segments Added

Figure 3.9: Short Range Saturday Proposed RTS Transit Network – Existing Segments Eliminated

Figure 3.10: Short Range Weekend Proposed RTS University of Florida Campus Transit Network

Figure 3.11: Short Range Weekend Proposed RTS University of Florida Campus Transit Network – New Segments Added

Figure 3.12: Short Range Weekend Proposed RTS University of Florida Campus Transit Network – Existing Segments Eliminated

Figure 3.13: Short Range Sunday Proposed RTS Transit Network

Figure 3.14: Short Range Sunday Proposed RTS Transit Network – New Segments Added

Figure 3.15: Short Range Sunday Proposed RTS Transit Network – Existing Segments Eliminated

  - Operating Requirements

Route level operating statistics have been calculated based on the first five years’ implementation of the COA service plan (short range). As noted earlier, the most significant increases occur on weekends.  Saturday hours and miles increase by 23% and 24% respectively while Sunday hours and miles are even more generously increased by 78% and 93%.  The programmed improvements to weekday service represent only a 5% increase in revenue hours and 4% increase in revenue miles over the five-year implementation period. Tables 3.3 through 3.6 provide a summary of daily and annual operating requirements, while Table 3.7 details route level requirements by day of week.

Table 3.3: COA Short Range Service Plan Daily Revenue Hours

Table 3.4: COA Short Range Service Plan Daily Revenue Miles

Table 3.5: COA Short Range Service Plan Annual Revenue Hours

Table 3.6: COA Short Range Service Plan Annual Revenue Miles

Table 3.7: COA Short Range Service Frequency and Operating Requirements by day of Week

Table 3.7: COA Short Range Service Frequency and Operating Requirements by day of Week (continued)

Note: Color coded cells reflect route interlines by time period

Table 3.7: COA Short Range Service Frequency and Operating Requirements by day of Week (continued)

Note: Color coded cells reflect route interlines by time period

Table 3.7: COA Short Range Service Frequency and Operating Requirements by day of Week (continued)

Note: Color coded cells reflect route interlines by time period

Long Range Service Plan

  - Long Range 6-10 Year Service Plan

The COA Long Range Service Plan is highlighted by route improvements in the Santa Fe College (SF) area as well as south Gainesville.  Service span improvements are also made to select routes, particularly on weekends as outlined in Table 4.1 below.

Table 4.1: COA Long Range Service Plan by Implementation Year

- Route Level Service Plan Recommendations

Following is a brief description of proposed Long Range service changes by route. Each description identifies proposed recommendations for changes to route alignment, service frequency, and span of service hours for weekday, Saturday and Sunday. Route descriptions are categorized as City Routes, UF Campus Routes and Later Gator Routes. Long-Range Service Plan (Years 6 to 10) does not include changes on the following routes: 9, 11, 13, 15, 21, 27, 28, 34, 35, 36, 38, 41, 46, 227 and UF Campus and Later Gator routes. Table 4.2 identifies proposed Long-Range Plan service hours and frequencies by day of week. Figures 4.1 through 4.15 depict the proposed Long-Range Service Plan Network, new segments and existing segments eliminated by day of week for system wide and UF Campus services.

City Routes

Route 1 – Downtown Station to Butler Plaza via Archer Road

Route Alignment Recommendations:

No alignment changes are recommended.

Service Frequency Recommendations:

Weekday: no change

Saturday: operate 30 minute service from start of service in morning through end of service in evening

Sunday: operate 30 minute service from start of service in morning through end of service in evening

Span of Service Recommendations:

Weekday: extend service 2 hours later

Saturday: extend service 2 hours later

Sunday: start service 2 hours earlier and extend 1 hour later

Route 2 – Downtown Station to NE Walmart Supercenter via Lincoln Estates

Route Alignment Recommendations:

No alignment changes are recommended.

Service Frequency Recommendations:

Weekday: operate 30 minute service from start of service in the morning, operate 60 minute evening and late evening (new) service

Saturday: new 60 minute service in evening hours

Sunday: no change

Span of Service Recommendations:

Weekday: extend service 3 hours later in the evening

Saturday: extend service 2 hours later in the evening

Sunday: no change

Route 5 – Downtown Station to Oaks Mall via University Avenue

Route Alignment Recommendations:

No alignment changes are recommended.

Service Frequency Recommendations:

Weekday: no change

Saturday: no change

Sunday: no change

Span of Service Recommendations:

Weekday: no change

Saturday: no change

Sunday: start service 2 hours earlier and extend 1 hour later

Route 6 – Downtown Station to North Walmart Supercenter via 6th Street

Route Alignment Recommendations:

No alignment changes are recommended.

Service Frequency Recommendations:

Weekday: improve day time frequencies from 60 to 30 minutes, maintain evening at 60 minutes

Saturday: no change

Sunday: no change

Span of Service Recommendations:

Weekday: Begin service 20 minutes later in the morning (6:00 a.m. versus 5:40 a.m.), extend service 1 hour and fifty minutes later in the evening hours

Saturday: begin 1 hour earlier in morning, operate 2 hours and fifty minutes later in the evening hours

Sunday: no change

Route 8 – Shands to North Walmart Supercenter via NW 13th Street

Route Alignment Recommendations:

No alignment changes are recommended.

Service Frequency Recommendations:

Weekday: improve late evening frequency from 45 to 90 minutes

Saturday: no change

Sunday: no change

Span of Service Recommendations:

Weekday: extend 1 hour later in evening

Saturday: no change

Sunday: no change

Route 10 – Downtown Station to Santa Fe via 16th/23rd Avenue

Route Alignment Recommendations:

No alignment changes are recommended.

Service Frequency Recommendations:

Weekday: no change

Saturday: service is recommended to be deleted

Sunday: no service

Span of Service Recommendations:

Weekday: no change

Saturday: service is recommended to be deleted

Sunday: no service

Route 12 – Reitz Union to Butler Plaza

Route Alignment Recommendations:

No alignment changes are recommended.

Service Frequency Recommendations:

Weekday: no change

Saturday: no change

Sunday: new 45 minute frequency in morning and evening hours

Span of Service Recommendations:

Weekday: no change

Saturday: no change

Sunday: operate 1 hour earlier in the morning and 1 hour later in the evening

Route 16 – UF / Shands to Sugar Hill

Route Alignment Recommendations:

The COA Study recommends interlining the Route 16 and 43 together through the UF Campus on Saturdays and Sundays.

Service Frequency Recommendations:

Weekday: improve evening service from 60 to 30 minute frequency

Saturday: no change

Sunday: no change

Span of Service Recommendations:

Weekday: reduced evening service by 1 hour

Saturday: extend 1 hour later in the evenings

Sunday: operate 1 hour earlier in the morning and extend 1 hour later in the evening

Route 17 – UF / Shands to Downtown Station

Route Alignment Recommendations:

The COA Study recommends modification of the routing on the western end of this route (same as Route 16). The proposed route alignment would operate west on SW 16th Avenue, north on SW 16th Street, east on SW Archer Road, north on Newell Drive, west on Museum Road, south on Center Drive, east on SW Archer Road, south on SW 16th Street, and east on SW 16th Avenue, continuing on existing routing.

Service Frequency Recommendations:

Weekday: improve evening service from 60 to 30 minute frequency

Saturday: no changes

Sunday: no changes

Span of Service Recommendations:

Weekday: reduce evening service by 1 hour

Saturday: no changes

Sunday: no changes

Route 20 – Reitz Union to Oaks Mall via 20th Avenue

Route Alignment Recommendations:

No alignment changes are recommended.

Service Frequency Recommendations:

Weekday: no changes

Saturday: no changes

Sunday: no changes

Span of Service Recommendations:

Weekday: extend late evening service by 30 minutes

Saturday: no changes

Sunday: operate 2 hours earlier in the morning and extend 2 hours later in the evening

Route 23 – Haile Market Square to Santa Fe via Fort Clarke

Route Alignment Recommendations:

The COA recommends a restructuring of Route 23. With the extension of Route 62 from the Oaks Mall to SF (see Route 62 proposed changes below), Route 23 is recommended to operate from the Haile Market Square to SF (assuming a portion of the existing Route 76 which is proposed to be eliminated). The new Route 23 would operate from the Haile Market Square plaza, travel east on SW 24th Avenue, north on SW 75th Street, west on West Newberry Road, north of Ft. Clarke Boulevard, east on NW 23rd Avenue, and north on NW 83rd Street to SF. Southbound routing would follow the reverse routing.

Service Frequency Recommendations:

Weekday: service would be reduced from 23 minutes during midday and afternoon peak to 40 minutes all day.

Saturday: no service

Sunday: no service

Span of Service Recommendations:

Weekday: reduce evening service by 3 hours

Saturday: no service

Sunday: no service

Route 24 – Downtown Station to Lamplighter / Job Corps

Route Alignment Recommendations:

No alignment changes are recommended.

Service Frequency Recommendations:

Weekday: no change

Saturday: no service

Sunday: no service

Span of Service Recommendations:

Weekday: Route 24A – Lamplighter: begin service 30 minutes later in morning and extend 1 hour later in the evening. Route 24B – Job Corps: begin service 1.5 hours later in the morning (low productivity)

Saturday: no service

Sunday: no service

Route 39 – Santa Fe to Santa Fe Downtown via 39th Avenue

Route Alignment Recommendations:

The COA Study recommends the following route alignment change to the Route 39. This route would start at SF, travel north on NW 83rd Street, west on N Road, north on NW 91st Street, east on NW 39th Avenue, south on NW 34th Street, south on NW 31st Boulevard, east on NW 31st Street / Glen Springs Road, south east on NW 23rd Boulevard / NW 23rd Avenue, south on NW 13th Street, west on NW 5th Avenue, south on SW 6th Street to the Downtown SF Campus.

Service Frequency Recommendations:

Weekday: improve service frequency from 60 to 35 minutes all day

Saturday: no service

Sunday: no service

Span of Service Recommendations:

Weekday: extend afternoon service 3 hours from 3:00 p.m. to 6:00 p.m.

Saturday:  no service

Sunday: no service

Route 40 – The Hub to NW 43rd Street

Route Alignment Recommendations:

The COA Study recommends the new Route 40 to operate between The Hub on UF Campus to NW 43rd Street. The route would depart The Hub, travel north on Buckman Drive, west on University Avenue, south on Gale Lemerand Drive, west on Stadium Road, north on SW 23rd Street, west on SW 2nd Avenue, north on NW 34th Street, west on NW 39th Avenue, north on NW 43rd Street to NW 53rd Avenue.

Service Frequency Recommendations:

Weekday: new route at 30 minute frequency all day

Saturday: no service

Sunday: no service

Span of Service Recommendations:

Weekday: new route operating between 6:30 a.m. and 6:00 p.m.

Saturday: no service

Sunday: no service

Route 43 – Shands to Santa Fe

Route Alignment Recommendations:

The COA Study recommends interlining the Route 16 and 43 together through the UF Campus on Saturdays and Sundays.

Service Frequency Recommendations:

Weekday: modify frequency from 28 to 30 minutes

Saturday: new 40 minute service during the day with 60 minutes in the early evening

Sunday: new 60 minute service all day

Span of Service Recommendations:

Weekday: no change

Saturday: new service between 7:00 a.m. and 7:30 p.m.

Sunday: new service between 9:00 a.m. and 6:40 p.m.

Route 53 – NW 43rd Street to North Walmart Supercenter / North US Hwy 441

Route Alignment Recommendations:

The COA Study recommends adding new Saturday service on the Route 53.

Service Frequency Recommendations:

Weekday: no change

Saturday: new service at 60 minutes all day

Sunday: no service

Span of Service Recommendations:

Weekday: no change

Saturday: new service between 8:00 a.m. and 5:00 p.m.

Sunday: no service

Route 62 – Santa Fe to Butler Plaza

Route Alignment Recommendations:

The COA Study recommends the following route alignment change to the Route 62. This route would start from the Butler Plaza Transit Stop, travel south on SW 35th Boulevard, southwest on SW Archer Road, north / west on SW 37th Boulevard, north on SW 42nd / 43rd Street, west on SW 20th Avenue, north on SW 62nd Boulevard, enter and serve the Oaks Mall Transit Stop, proceed north on NW 62nd Street, east on Newberry Road, north on NW 55th Street, west on NW 23rd Avenue, north on NW 83rd Street to the SF Campus Bus Stop.

Service Frequency Recommendations:

Weekday: improve service frequency from 60 to 20 minutes all day

Saturday: no service

Sunday: no service

Span of Service Recommendations:

Weekday: extend service 7 hours later from 3:00 p.m. until 10:00 p.m.

Saturday: no service

Sunday: no service

Route 75 – Oaks Mall to Butler Plaza via 75th Street

Route Alignment Recommendations:

No alignment changes are recommended.

Service Frequency Recommendations:

Weekday:  no change

Saturday: no change

Sunday: no change

Span of Service Recommendations:

Weekday: extend service 3 hours later from 8:00 p.m. until 11:00 p.m.

Saturday: no change

Sunday: no change

Route 76 – Santa Fe to Halle Market Square

Route Alignment Recommendations:

The existing Route 76 connects Halle Market Square with Santa Fe College.  The COA Study proposes the elimination of this route and its replacement with a modified Route 23 (see description above).

Service Frequency Recommendations:

Weekday: route eliminated and replaced by modified Route 23

Saturday: no service

Sunday: no service

Span of Service Recommendations:

Weekday: route eliminated and replaced by modified Route 23

Saturday: no service

Sunday: no service

Route 77 – Santa Fe to Cabana Beach

Route Alignment Recommendations:

The existing Route 77 connects Cabana Beach with Santa Fe College.  The COA Study proposes the elimination of this route and its replacement with a modified Route 62 (see description above).

Service Frequency Recommendations:

Weekday: route eliminated and replaced by modified Route 62

Saturday: no service

Sunday: no service

Span of Service Recommendations:

Weekday: route eliminated and replaced by modified Route 62

Saturday: no service

Sunday: no service

Table 4.2: Long Range Plan Proposed Service Hours and Frequencies

Table 4.2: Long Range Plan Proposed Service Hours and Frequencies (continued)

Note: No changes proposed in Long-Range Plan for UF Campus and Later Gator routes.

Figure 4.1: Long Range Weekday Proposed RTS Transit Network

Figure 4.2: Long Range Weekday Proposed RTS Transit Network – New Segments Added

Figure 4.3: Long Range Weekday Proposed RTS Transit Network – Existing Segments Eliminated

Figure 4.4: Long Range Saturday Proposed RTS Transit Network

Figure 4.5: Long Range Saturday Proposed RTS Transit Network – New Segments Added

Figure 4.6: Long Range Saturday Proposed RTS Transit Network – Existing Segments Eliminated

Figure 4.7: Long Range Sunday Proposed RTS Transit Network

Figure 4.8: Long Range Sunday Proposed RTS Transit Network – New Segments Added

Figure 4.9: Long Range Sunday Proposed RTS Transit Network – Existing Segments Eliminated

  - Operating Requirements

Route level operating statistics have been calculated based on the final five years’ implementation of the COA service plan (long range).  Weekend service hours and miles continue to dominate the service improvements, though not as greatly as the short range service plan.  Saturday hours and miles increase by 11% and 9% respectively over the short range plan while Sunday hours and miles are slightly higher, increasing by 16% and 12%.  The programmed improvements to weekday service represent only a 5% increase in revenue hours and 6% increase in revenue miles over the short range plan. Tables 4.3 through 4.6 provide a summary of daily and annual operating requirements as well as a comparison between existing, Short-Range and Long-Range Service Plans. These tables are then followed by Table 4.7 which details route level requirements by day of week.

Table 4.3: COA Long Range Service Plan Daily Revenue Hours

Table 4.4: COA Long Range Service Plan Daily Revenue Miles

Table 4.5: COA Long Range Service Plan Annual Revenue Hours

Table 4.6: COA Long Range Service Plan Annual Revenue Miles

Table 4.7: COA Long Range Service Frequency and Operating Requirements by day of Week

Table 4.7: COA Long Range Service Frequency and Operating Requirements by day of Week (continued)

Note: Color coded cells reflect route interlines by time period

Table 4.7: COA Long Range Service Frequency and Operating Requirements by day of Week (continued)

Note: Color coded cells reflect route interlines by time period

Table 4.7: COA Long Range Service Frequency and Operating Requirements by day of Week (continued)

Note: Color coded cells reflect route interlines by time period

COA Vehicle and Facility Needs

The short and long range phases of the COA service plan include a mix of service improvements that will increase annual service hour delivery.  While some of these improvements involve increased service span and weekend operation, which has no impact on peak bus requirements, there are several frequency improvements and new service areas that will necessitate an increase to RTS’ fleet.  In addition, increased emphasis is placed on transfer locations outside of the Rosa Parks Transit Center.  While these secondary hubs will not be to the scale of Rosa Parks Transit Center, it is still important that they be seen as safe and inviting to the transit rider.  The following section outlines these capital needs.

  - COA Vehicle Needs

Currently, RTS operates 101 fixed-route buses during weekday peak service.  On Saturdays, this requirement is drastically curtailed to only 21 buses.  Sunday service bus requirement is reduced even more too only 14 buses.  In the short range service plan, greater emphasis is placed on improving off-peak and weekend services.  Thus, the overall peak requirement is only increased by three buses to a total of 104.  The long range plan sees modest growth in Saturday and Sunday service (plus two and plus one bus, respectively).  However, weekday service is increased by another four buses for a total of 108 fixed-route vehicles in peak operation.

Table 5.1: COA Peak Vehicle Requirements

  - Transit Passenger Facility Needs

Appropriately scaled transit facilities are nearly as important to the success of a service as the bus operations themselves.  Presently, RTS operates from two primary transfer facilities – the Rosa Parks Transit Center just south of downtown Gainesville and Reitz Union which is located near the central core of the UF campus.  Secondary transit centers or “superstops” are also employed throughout the service area including Waldo Road Walmart, North Gainesville Walmart, Santa Fe College, Oaks Mall and Butler Plaza.  The newer locations, such as the two Walmarts, have better infrastructure to include dedicated pull-out bays and passenger shelters.  Others are less formalized and may only offer shelter or a stand-alone bench.  To ensure passenger safety and comfort as well as an overall positive transit experience, it is important to invest in these transfer points.  Trip behavior gathered from the on-board survey highlighted the need for a number of realignments, which, in turn, emphasized the current deficiency in transfer facilities. To support the rerouting and improve accessibility and frequencies, transfer facilities are being proposed at UF, SF, the Five Points area, NW 13th St, Butler Plaza, and the Oaks Mall.

The Oaks Mall in west Gainesville is of particular concern.  This location experiences some of the highest volumes of passenger activity; yet the infrastructure is extremely understated.  Future improvements should include a defined (curbed or bollard separated) drive aisle for bus traffic that protects waiting passengers and restricts general purpose traffic traveling through the mall parking lot.  Additional seating and shelter space should also be considered.

In southwest Gainesville, a new Walmart supercenter is scheduled for development in the vicinity of Southwest 42nd Street & 24th Avenue.  Once opened, this new Walmart will replace the older location at Butler Plaza.  As part of the development plan, a new transit center is scheduled for construction.  This facility will provide dedicated passenger waiting and bus staging areas that are currently unavailable at the Butler Plaza location.  Consequently, buses in the area will be rerouted and all connection activity will be relocated to the new Walmart when opened.

As service in northern portions of Gainesville is restructured, the need for a new transit center becomes more defined in the vicinity of Northwest 13th Street and 23rd Avenue.  A Lowes home improvement center is located in the northwest quadrant of the intersection while a nearly vacant shopping center is situated across the street.  This location appears to offer the best opportunity for future redevelopment and a potential transit center.  RTS staff has recently reached out to the property owner about potentially developing the site and they were uninterested at this time.

Potential Transit Center

  - New Maintenance Facility

RTS is in the midst of constructing a new bus fleet maintenance and operations facility on Veitch Street, located approximately one-half mile south of the existing facility. The 140,000-square-foot facility is designed to accommodate 200 buses and will include dispatch, administrative and maintenance facilities including 16 drive-through bus bays.  By comparison, the existing eight-bay facility built in 1975 was built to accommodate 50 buses.  The new bus fleet maintenance and operations facility is scheduled to open at the end of 2014.

5.4	Scheduling Software / CAD/AVL Needs

The COA Study has also identified additional needs related to scheduling and monitoring RTS transit services. The current bus route scheduling practice requires an upgrade to existing state of the art software. Use of this type of scheduling software package would assist the scheduling staff in performing the job more time efficient and cost effective. Scheduling software programs provide an opportunity to optimize schedule writing, trip blocking and run cutting, often resulting in cost savings to the transit agency both short and long term.  Additionally, the implementation of a Computer Aided Dispatch/ Automatic Vehicle Locator (CAD/AVL) System would help RTS efficiently manage vehicle operations and vehicle location identification.  The current AVL system has limited functionality, simply showing bus position and not communicating more pertinent information such as on-time performance, hard braking, bus bunching, etc.